GENERATION AND ANALYSIS OF FREQUENCY MODULATED (FM) SIGNALS USING MATLAB

OBJECTIVES:

- To generate a Frequency Modulated (FM) signal using MATLAB by varying the frequency of a high-frequency carrier in accordance with the amplitude of the message signal and to observe the resulting waveform in the time domain.

- To analyze the frequency spectrum of the generated FM signal using FFT and identify the carrier component, sideband frequencies, and the bandwidth of the FM signal based on Carson's rule.

OBJECTIVE 1: To generate a Frequency Modulated (FM) signal using MATLAB by varying the frequency of a high-frequency carrier in accordance with the amplitude of the message signal and to observe the resulting waveform in the time domain.

PROGRAM:

clc;

clear;

close all;

% Parameters

Am = 1;

Ac = 1;

fm = 5;

fc = 50;

beta = 5;

fs = 1000;

% Time vector

t = 0:1/fs:1;

% Message signal

m = Am * cos(2*pi*fm*t);

% Carrier signal

c = Ac * cos(2*pi*fc*t);

% FM signal

fm_sig = Ac * cos(2*pi*fc*t + beta*sin(2*pi*fm*t));

% Plot signals

figure('Name','Frequency Modulation (FM)','NumberTitle','off');

subplot(3,1,1)

plot(t,m)

title('Message Signal')

xlabel('Time (s)')

ylabel('Amplitude')

grid on

subplot(3,1,2)

plot(t,c)

title('Carrier Signal')

xlabel('Time (s)')

ylabel('Amplitude')

grid on

subplot(3,1,3)

plot(t,fm_sig)

title('FM Signal')

xlabel('Time (s)')

ylabel('Amplitude')

grid on

% Display parameters

fprintf('Message Frequency = %d Hz\n', fm);

fprintf('Carrier Frequency = %d Hz\n', fc);

fprintf('Modulation Index (β) = %.2f\n', beta);

OUTPUT:

OBJECTIVE 2 : To analyze the frequency spectrum of the generated FM signal using FFT and identify the carrier component, sideband frequencies, and the bandwidth of the FM signal based on Carson's rule.

PROGRAM:

clc;

clear;

close all;

%% Input Parameters

Am = 2;              % Message signal amplitude

fm = 500;            % Message frequency (Hz)

Ac = 1;              % Carrier amplitude

fc = 10000;          % Carrier frequency (Hz)

kf = 1000;           % Frequency sensitivity (Hz/V)

fs = 100000;         % Sampling frequency (Hz)

T = 0.02;            % Simulation duration (s)

t = 0:1/fs:T;

%% Message Signal

m = Am * cos(2*pi*fm*t);

%% Frequency Deviation

delta_f = kf * Am;

%% Modulation Index

beta = delta_f / fm;

%% FM Signal

fm_signal = Ac * cos(2*pi*fc*t + beta * sin(2*pi*fm*t));

%% Plot Message and FM Signals

figure;

subplot(2,1,1)

plot(t,m,'LineWidth',1.5)

title('Message Signal')

xlabel('Time (s)')

ylabel('Amplitude')

grid on

subplot(2,1,2)

plot(t,fm_signal,'LineWidth',1.5)

title('Frequency Modulated Signal')

xlabel('Time (s)')

ylabel('Amplitude')

grid on

%% Spectrum Analysis

N = length(fm_signal);

FM_fft = fftshift(abs(fft(fm_signal))/N);

f = (-N/2:N/2-1)*(fs/N);

figure

plot(f,FM_fft,'LineWidth',1.5)

title('FM Signal Spectrum')

xlabel('Frequency (Hz)')

ylabel('Magnitude')

grid on

xlim([fc-10000 fc+10000])

%% Display Results

fprintf('\nFM SIGNAL PARAMETERS\n');

fprintf('---------------------------\n');

fprintf('Carrier Frequency      = %.0f Hz\n',fc);

fprintf('Message Frequency      = %.0f Hz\n',fm);

fprintf('Message Amplitude      = %.2f V\n',Am);

fprintf('Frequency Sensitivity  = %.0f Hz/V\n',kf);

fprintf('\nCALCULATED VALUES\n');

fprintf('---------------------------\n');

fprintf('Frequency Deviation (Δf) = %.0f Hz\n',delta_f);

fprintf('Modulation Index (β)     = %.2f\n',beta);

%% Carson's Rule Bandwidth

BW = 2 * (delta_f + fm);

fprintf('Bandwidth (Carson Rule) = %.0f Hz\n',BW);

OUTPUT: